Documentation File for B0 measurements

18.11.2025

- Making first measurements with phyphox:

  - Depending on position, like vicinity to other electrical devices, or measuring devices themselves (Louis or Bens phone) results differ drastically

    - While sometimes earth magnetic field (around 40uT, vertical) seem somewhat represented, reproducibility is lacking

  - This seems to boil down to following problems:

    - Depending on how isolated a phone is, measurements get better or worse

    - Putting phone in airplane mode should improve measurements

    - Calibration of phyphox leads to varitations in the absolute values measured, for measuring stronger magnetic fields of coils (mT range) it seems advisable to not use calibration and instead make a background measurement

      - => short measurement, substract mean value as we do not want to override noise

  - It should also be noted that measuring center of a magnetic field will become difficult, as sensor in phone might not be at right position and different phones might also differ in positions of sensors

    - While this will lead to different results altogether, in terms of absolute values, fluctuations should/might still be similar

- Simple coil:

  - Nothing measured yet, either problems with phone or coil itself

19.11.2025

- We did 3 measurements with 3 different devices on the same spot, see README in data/raw

- How to deal with different sampling rates

  - 50Hz to two times 100Hz

- Analyze histogram/gaussian model -> instead of curvefit I will calculate the parameters directly out of the data (fitting to a histogram feels unnecessarily complicated and like I just loose information)

  - What are the stddev?

  - Can we resimulate the model, will it be the same?

  - Can we find correlations between the models

- Analyze the allan deviation

  - How good is a gaussian model even

  - From the data it seems like I get a more or less steady decline, although not as good as with simulated data based on gaussian (nice -1/2 slope in log)

20.11.2025

- Goals For the day (so far)

  - ! Make another long term measurement (5-10min) to see how allan deviation evolves

    - Can I see a better curve, does the slope go up -> systematic errors

    - Small comparison measurement not necessary, can just cut data short!

  - Find out

    - How can I compare three measurements from three devices, correlation??

    - I decided to compare the PSD. Depending on where I find peaks, I can characterize specific noise components

      - As I did not measure at the same time, a likeliness function doesn’t seem reasonable. Even environmental contributions might differ at different instances, which is why I don’t think they will be very comparable. A PSD seems more flexible in this regard while granting similar informations.

  - Repeating measurement from yesterday to check for a) reproducabitlity and b) if the PSD shows a similar characteristic

  - ! Meassure setup with artificial magnetic field (timescale based on these ideas)